# Highways and Engineering Service

# Economy and Environment

Consultation Response on behalf of Head of Highways and Engineering

Application: 25-00971-PIP

Date Received:      27 October 2025

Date of Response: 04 November 2025

Proposal:

Application for permission in principle for residential development of up to 8 dwellings.

Consultation Report:

The application site is located within a residential area. There is few local amenities in walking distance although there is some public transport links available. We would not consider this to be a particularly sustainable location.

The traffic associated with this proposal is unlikely to impact upon the local transport network.

The level of parking provided should be 200% per unit.

The proposed site access is existing and inadequate for a development of this scale and use. Typically would expect an access supporting a development of units or more to be adopted by the local authority. This would require an access of 5.5m in width with 2 x 2m footways. This could due to the low unit numbers be reduced to a carriageway of 3.5m over a short length with a 1.2m footway to one side of the access road. There is however insufficient space to accommodate such an access. How this is dealt with should be the main highways consideration of this proposal. The highway needs to consider emergency vehicle access/refuse collection and pedestrian access without conflict or suitable mitigation made. This access point is a road safety concern.

No changes to the existing highways network are proposed as a result of this development.

Any new highways constructed as part of this development should be done so to an adoptable standard. This will require a 2m footway fronting any units and a 5.5m carriageway. There will also be  a need to accommodate refuse vehicles turning within the site. We also have concerns about the ground conditions at this location due to the presence of a stream/spring running through the site.

No PROW will be obstructed by this proposal.

Amendments to Consultation Response:

No amendments to report

Officer Recommendation:

We have several concerns about this site being considered for residential purposes but if those concerns can be addressed suitably we would not object to this proposal in principle.

Highways Sign off

John Gillighan 04/11/2025