Proposed Landscaping

Ref: SR.141124.A

Lawned area

•	All planting operations shall comply with BS 4428 (1989) & AMD, Code of Practice for General Landscaping Operations

•	Total minimum soil depths: 150 mm topsoil, minimum 450 mm subsoil

•	Subsoil shall conform to BS 8601:2013 Specification for Subsoil and             requirements for use.

•	Topsoil conforming to B.S. 3882: 2015 shall be multi-purpose soil of uniform composition throughout, of light to medium texture, and shall be capable of being broken down to a fine tilth.

•	Lawn with rye grass mix shall be:

PM51 Greenscape by Olivers seed www.oliver-seeds.co.uk or similar comprising:

40% Double, 4turf tetraploid ryegrass

30% Maxima, Strongcreeping red fescue

30% Esquire, Perennial ryegrass

•	Sowing rate: seed at a rate of 35 – 50 g per m2

Hedge planting

Hedge planting as shown on the plan and on the inside of the 1.5m stone boundary wall.

Whip planting details

| % | Botanic | Common | Nursery size |
| --- | --- | --- | --- |
| 50 | Crataegus monogyna | Hawthorn | Min 750mm |
| 50 | Prunus spinosa | Blackthorn | Min 750 mm |

Planting to be in two rows 1 metre apart and at 0.75 metre centres, staggered.

Maintenance

Any plants failing within first 5 years to be replaced by same