BIODIVERSITY NET GAIN ASSESSMENT

SUMMARY REPORT

LAND AT HANSON SPRINGS, ROCHDALE

Biodiversity Net Gain Assessment

Summary Report

Land at Hanson Springs, Rochdale

A report for

Hanson Springs,

Gorrells Way,

Rochdale

OL11 2PX

Report by

PENNINE ecological

24 The Highgrove

Heaton

Bolton

BL1 5PX

Tel: (01204) 844545

email: bob@pennineecological.co.uk

web:  www.pennineecological.co.uk

Report Author:

Hugh Richards, MSci

Reviewer:

Robert N. Leatham B.Sc. (Hons.), P. dip.

October 2024

ConteNTS

1.	Introduction	1

1.1	Background	1

1.2	Site Context	1

2.	Biodiversity Net Gain Results	3

3.	BNG Conclusion	6

4.	BNG Proposal	7

References	8

Appendix A: Figures	9

Terms of Use

This report has been compiled by Pennine Ecological with all reasonable skill, care and diligence within the terms of the instruction and permissions granted by the client. The results, conclusions and recommendations of this report are proportionate and in line with the British Standard 42020:2013.

This is a technical report and does not represent legal advice/opinion.

The report is for the sole use of the commissioning client in connection with the development project described in the report, and must not be used for any other purpose, copied, re-produced or sent to any other party other than the Local Planning Authority without the permission of Pennine Ecological.

This report remains the property of Pennine Ecological and cannot be relied upon until full payment has been made. Pennine Ecological reserve the right to retract any survey reports submitted to planning where payments are outstanding.

Pennine Ecological will retain the right to re-publish data obtained, and to forward data collected during all its ecology surveys to the local wildlife records centre.

# Introduction

## Background

This report provides the results of the BNG Assessment for the proposed construction of a new building, the extension of an existing building, and associated infrastructure, at Hanson Springs, Rochdale.

The information provided within this report should be read in conjunction with the following documents:

- Pennine Ecological (2024a). 004 Hanson Springs dv2 - The_Statutory_Biodiversity_Metric_Calculation_Tool_-_Macro_enabled_02.24

- Pennine Ecological (2024b). 004 Hanson Springs dv2 - Statutory_Biodiversity_Metric_Condition_Assessments-_Feb24

- Hawthorn Design Partnership (2024) Location Plan. (Drawing No.: 188-PLN-L-16-A).

- Hawthorn Design Partnership (2024) Block Masterplan. (Drawing No.: 188-PLN-L-10-B).

## Site Context

The site is situated in the outskirts of Rochdale, within a primarily urban setting. Green corridors along the railway and Rochdale Canal are present to the northwest and southeast, respectively. The site’s central National Grid Reference is SD 8895 1153.

The site is shown below, outlined in red.

Figure 1.1: Land at Hanson Springs, Rochdale

# Biodiversity Net Gain Results

### Habitat BNG Results

The biodiversity calculation concluded that the current proposal would result in a biodiversity net gain in habitat units and would satisfy trading rules.

The current proposal would result in a 0.12 habitat unit gain / 12.28% net gain. A summary of the habitat calculations is provided in Tables 2.1 and 2.2 below.  Refer also to Appendix A, Figures 1 and 2.

Table 2.1: Habitat baseline calculations

| Habitat | Habitat Condition | Habitat Area (ha) | Habitat Unit Value | Area Retained (ha) | Area Enhanced (ha) | Habitat Unit Loss |
| --- | --- | --- | --- | --- | --- | --- |
| Bare ground | Poor | 0.008677 | 0.02 | 0 | 0 | 0.00 |
| Developed land; sealed surface | N/A - Other | 0.607274 | 0.00 | 0.607274 | 0 | 0.00 |
| Developed land; sealed surface | N/A - Other | 1.517889 | 0.00 | 1.333286 | 0 | 0.00 |
| Modified grassland | Poor | 0.051559 | 0.10 | 0 | 0.051559 | 0.00 |
| Modified grassland | Moderate | 0.005833 | 0.02 | 0.005833 | 0 | 0.00 |
| Other woodland; broadleaved | Moderate | 0.026730 | 0.21 | 0.02673 | 0 | 0.00 |
| Other neutral grassland | Moderate | 0.024280 | 0.19 | 0 | 0 | 0.19 |
| Ruderal/Ephemeral | Moderate | 0.004913 | 0.02 | 0 | 0 | 0.02 |
| Urban tree | Moderate | 0.041 | 0.38 | 0.041 | 0 | 0 |
| TOTAL | - | 2.25 (excl. urban tree) | 0.95 | 2.01 | 0.05 | 0.22 |

Table 2.2: Proposed / created on site habitats

| Habitat | Area (Ha) | Distinctiveness | Condition | Strategic Significance | Standard Time to Target Condition (Years) | Habitat Units Delivered | Assessment Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Developed land; sealed surface | 0.2084 | V.Low | N/A | Area/compensation not in local strategy/ no local strategy | 0 | 0 | New building and building extension. |
| Urban tree | 0.0082 | Medium | Moderate | Formally identified in local strategy | 27 | 0.03 | Young native tree planting (two trees) in grassland enhancement area. |
| Introduced shrub | 0.0086 | Low | N/A | Area/compensation not in local strategy/ no local strategy | 1 | 0.02 | Ornamental mixed-species planting beneath line of trees at eastern boundary. |
| TOTAL | 0.22 |  |  |  |  | 0.05 |  |

2.2.1	  Linear Habitat (Hedgerow) Results

The biodiversity calculation concluded that the current proposal would result in neither net loss nor gain in linear (hedgerow) units, as no new hedgerow creation is included under current proposals.

A summary of the habitat present on-site is provided in Table 2.3 below.  Refer also to Appendix A, Figures 1 and 2.

Table 2.1: Habitat baseline calculations

| Habitat | Habitat Condition | Length (km) | Habitat Unit Value | Length Retained (ha) | Length Enhanced (ha) | Habitat Unit Loss |
| --- | --- | --- | --- | --- | --- | --- |
| Line of trees | Poor | 0.058737 | 0.12 | 0.058737 | 0 | 0 |
| Line of trees | Moderate | 0.020247 | 0.08 | 0.020247 | 0 | 0 |
| TOTAL | - | 0.058737 | 0.12 | 0.058737 | 0 | 0 |

# BNG Conclusion

The current proposals would result in an overall net gain with all habitat trading rules satisfied. The headline results taken from the BNG assessment (Pennine Ecological, 2024a), can be found below.

Table 3.1: Summary of BNG Assessment taken from BNG calculation tool headline results.

# BNG Proposal

The proposal will result in the loss of small areas of other neutral grassland and ruderal / ephemeral habitat to the southwest and southeast of the proposed new building, respectively.

To compensate for this loss and deliver a 10% net gain in habitat units, an area of modified grassland at the site’s northern boundary will be enhanced from poor condition modified grassland to good condition other neutral grassland. Furthermore, two young native trees will be planted in this area. These habitats will deliver 0.41 and 0.03 habitat units, respectively.

To deliver good condition other neutral grassland, the area (currently supporting poor condition modified grassland) will be scarified, rotavated, and covered with opaque tarpaulin to prevent regrowth of modified grassland species. The area will then be sown with an appropriately diverse wildflower seed mix. Sufficient quantities of seed will be used to establish in excess of 10 species per square metre. Sufficiently regular mowing will take place to maintain a varied sward height throughout spring and summer. At the end of the growing season, the entire grassland will be mown and arisings removed. Presence of scrub, bracken, and invasive species will be controlled.

The small gains required to meet the hedgerow units target can be accomplished through new tree planting within the poor condition line of trees, reducing extent of the gaps, and thereby elevating its condition to at least moderate.

# References

Baker, J. Hoskin, R. Butterworth, T. (2019). Biodiversity Net Gain. Good practice principles for development. Part A: A practical guide.

Department for Environment, Food & Rural Affairs (DEFRA) (2023) - The Statutory Biodiversity Metric: User Guide (draft). Date: February 2024.

Hawthorn Design Partnership (2024) Block Masterplan. (Drawing No.: 188-PLN-L-10-B).

Hawthorn Design Partnership (2024) Location Plan. (Drawing No.: 188-PLN-L-16-A).

Pennine Ecological (2024a). 004 Hanson Springs dv2 - The_Statutory_Biodiversity_Metric_Calculation_Tool_-_Macro_enabled_02.24

Pennine Ecological (2024b). 004 Hanson Springs dv2 - Statutory_Biodiversity_Metric_Condition_Assessments-_Feb24

UKHab Ltd (2023). UK Habitat Classification Version 2.0 (at https://ukhab.org/)

Appendix A: Figures

Figure 1: Existing Habitat Plan

Figure 2: Proposed Site Plan