Landscape and ecological management plan

with recommended ecological planting scheme – Aspinall Street, Heywood

1. Introduction

Living Ecosystems was commissioned by Alan Craigen in June 2023 to  produce a Landscape and Ecological Management Plan for a proposed development on a plot of land at the junction of Townrow Street and Aspinall Street, Heywood, Rochdale OL10 4FH. The site is located at  grid reference SD 86018 10850.

Location of proposed development and it's surroundings

The site is located  on Aspinall Street in Heywood. The site is approximately 15 metres wide (west to east) by 50 metres long (north to south) or 750 square metres hectares in extent. The site is bounded by Aspinall Street  to the east, and Townrow to the south. To the north are residential properties; with residential development to the south west and scrub to the north west. Currently the site is occupied by B4 Improved grassland dominated by tall rank grassland.

The development site as it is currently.

2. Development Proposal

The proposed development and its associated works include a residential scheme of three bungalows, with related landscaping and infrastructure.

The proposal is for the construction of three bungalows on a plot off waste-land on Aspinall Street.

Planning permission has been granted by Rochdale Borough Council for the development subject to planning condition is as follows:-

No above ground works shall take place until a scheme of biodiversity enhancements for the site including, but not limited to, provision for bird boxes and bat boxes, has been submitted to and approved in writing by the Local Planning Authority. No dwelling shall be occupied until such time as the duly approved biodiversity enhancement measures within that plot have been installed in accordance with the duly approved detail. Reason: In the interests of enhancing biodiversity and habitat provision within the site in accordance with Policy G7 of the adopted Rochdale Core Strategy and the National Planning Policy Framework.

(a) Notwithstanding the details shown on the approved plans, the development hereby permitted shall not be occupied until full details of both hard and soft landscaping works have been submitted to and approved in writing by the Local Planning Authority. The details shall be substantially in accordance with the submitted plan ref. C393-001/D, and shall include the formation of any banks, terraces or other earthworks, hard surfaced areas and materials, planting plans, specifications and schedules (including planting size, species and numbers/densities), existing plants / trees to be retained, a scheme for the timing / phasing of implementation works. Page 4 of 5 (b) The landscaping works shall be carried out in accordance with the approved scheme for timing / phasing of implementation and shall be maintained in accordance with the duly approved Landscape Environmental Management Plan. (c) Any trees or shrubs planted or retained in accordance with this condition which are removed, uprooted, destroyed, die or become severely damaged or become seriously diseased within 5 years of planting shall be replaced within the next planting season by trees or shrubs of similar size and species to those originally required to be planted. Reason: To ensure that the site is satisfactorily landscaped having regard to its location and the nature of the proposed development and in accordance with Policies P1, P2, P3, G6 and G7 of the adopted Rochdale Core Strategy and the National Planning Policy Framework.

The following report is intended to provide mitigation for any impacts on protected habitats and species at the site.

3. Pre-existing information about protected species at the site

Desktop Study and Data Search

The following sources of information and ecological records were consulted:

A.  MAGiC Maps: A web-based interactive map which brings together geographical, landscape and environmental information from Government sources at all levels.

B. The NBN Atlas: which is the UK’s largest repository of publicly available biodiversity data.

C. All measurements within this report are approximate only, and have been estimated whilst on site or calculated using mapping software (QGIS) or internet-based mapping services such as MAGiC Maps and Google Earth.

Evaluation methods of the data search

The habitats, vegetation and animal life were evaluated with reference to standard nature conservation criteria as described in A Nature Conservation Review (Ratcliffe, 1977) and Guidelines for the Selection of Biological SSSIs (Bainbridge, et al., 2013). These are size (extent), diversity, naturalness, rarity, fragility, typicality, recorded history, position in an ecological or geographical unit, potential value and intrinsic appeal.

Habitats have been assessed to determine whether they meet those described in UK Biodiversity Action Plan: Priority Habitat Descriptions (Maddock, A (ed), 2008); these lists are used to help draw up the statutory lists of Priority Habitats, as required under Section 41 of the Natural Environment and Rural Communities (NERC) Act 2006. Where suitable, the ecological value of the habitats present has been assessed using the terms outlined in Guidelines for Ecological Impact Assessment in the UK and Ireland: Terrestrial, Freshwater, Coastal and Marine (CIEEM, 2018).

Government advice on wildlife, as set out in the National Planning Policy Framework (Ministry of Housing, Communities and Local Government, 2021) and associated government circulars has been taken into consideration. Legislation relating to protected species, such as those listed under Schedules 1, 5, 6 and 8 of the Wildlife and Countryside Act 1981 (as amended) and The Conservation of Habitats and Species Regulations 2017, is referenced where applicable, and any impacts to protected species are evaluated in accordance with current guidance.

The presence of any Priority Species, as listed under Section 41 of the Natural Environment and RuralCommunities (NERC) Act 2006 is noted, and habitats are assessed in terms of their suitability and valuefor these species. The presence of habitats and/or species listed by the Greater Manchester Biodiversity  Action Plans has been taken into account in the evaluation of the site environmental schemes and designations, including details of statutory nature conservation sites;

A. Greater Manchester Local Record Centre (GMLRC);

B. Greater Manchester Biodiversity Action Plan (BAP);

3.1 Desktop Study and Data Search Results

The site is not designated as a statutory designated site for nature conservation, and no statutory designated sites for nature consrvation lie within a 1.0  kilometre radius of the site.

The site has no non-statutory designations for nature conservation, and no non-statutory designated sites for nature conservation lie within a 1.o kilometre radius of the site.

The relevant protected species records from the ecological data search  within the 500metre radius search area were as follows;

Common Pipistrelle (Pipistrellus pipistrellus) a single roost site was recorded in Queen's Park, Heywood  approximately 0.2 km to the West of the site in 2015. (Pipistrellus pipistrellus is a priority species designated in ECH4, LBAP, UKBAP, WCA5).

Hedgehogs (Erinaceus europaeus) have been recorded 6 times within 1.0 kilometres of the site in the last 6 years. (Erinaceus europaeus is a priority Species, UKBAP, LBAP, WCA5. IUCN Red List for British Mammals - vulnerable to extinction).

4. Summary of existing habitats at the Aspinall Street site

B4 Improved grassland dominated by tall rank grassland.

A detailed habitat survey of the site was undertaken on the morning of 18th June 2023. The survey followed the Phase 1 Habitat Survey Methodology (JNCC, 2010). This involves walking over the site, mapping and target noting any semi-natural habitats. The survey area includes the footprint of the proposed development.The weather was slightly overcast, dry and warm (22 degrees C) with a light wind. The survey was carried out by Richard Parkinson a very experienced Principal Ecologist with over 30 years of appropriate experience. The initial survey was carried out at an optimal time and I am entirely confident that the survey has identified and addressed all the relevant ecological issues at the site.

The following is a record of the species recorded within the site boundary during the survey.

Perenial ryegrass (Lolium perenne) A

False oat grass (Arrhenatherum elatius) F

Annual meadow-grass (Poa annua) O

Cock’s-foot Dactylis glomerata) A

Curled dock (Rumex crispus) O

Common dandelion (Taraxacum officinale) O

Common nettle (Urtica dioica) O

Common ragwort (Senecio jacobaea) O

Bramble (Rubus fruticosus) R

Elder (Sambucus nigra) R

Meadow buttercup (Ranunculus acris) O

White clover (Trifloium repens) O

Laurel (Laurus sp) O

5. Enhancement and management of habitats

Suggested Habitat enhancements

Good horticultural practices should be employed to ensure no, or minimal, use of pesticides and herbicides. The creation of up to 30% of the site as gardens and hedgerow post development will significantly improve the site. Gardens in the UK generally contain at least 15 native/useful species on average. Where possible landscape planting within the proposed development area should aspire to consist of at least 50% of native or otherwise beneficial species preferably of UK provenance.  A beneficial species is one which though not native is not invasive or harmful; and also provides significant ecological resources to native species, such as nectar, fruit, berries, nesting opportunities or cover amongst others. This will contribute to local Biodiversity Action Plan goals. Beneficial species include non native species which are not invasive and provide significant wildlife opportunities such as nectar berries, cover or nesting opportunities. Details of the habitat improvements are contained in section 6.0 below (6.0 Defra Biodiversity Metric calculations for the proposed development at Aspinall Street in Heywood with required habitat improvements and planting instructions).

Enhancements for bats

Under the National Planning Policy Framework (2012) there is a requirement to provide features in proposed developments that promote biodiversity and habitat enhancements. The propose would have no signficant impact on bats.

It is therefore recommended: that new roosting opportunities for bats be provided at the site as part of the proposals.

A total of two bat roost boxes could be incorporated into or onto the walls of two of the bungalows as high up the wall as is practicable.

Suitable bat roosting boxes include the following:

Schwegler 1FQ bat box

EcoSurv Bat Box - Bespoke Brick
Code: 002134

Enhancements for birds

Under the National Planning Policy Framework (2012) there is a requirement to provide features in proposed developments that promote biodiversity and habitat enhancements. Consideration could be given to the incorporation of some of the following nest boxes into the development. All of the boxes are reasonably priced long lasting, durable, maintenance free Schwegler bird box designs which can be easily attached to the outside of buildings. Other brands are also available with similar specifications which will also be suitable.

Given that House Sparrows are known to be present in the vicinity of the site, it is recommended that new nesting opportunities for these UK BAP species be provided at the site.

House sparrows are listed as a Species of Principal Importance in England under Section 41 of the Natural Environment and Rural Communities (NERC) Act (2006), they are UK BAP Priority Species. House Sparrow is listed upon the RSPB’s Red List of ‘Birds of Conservation Concern 3’ (2009).

In order to provide new nesting opportunities for House Sparrow at the site, it is recommended that a single House Sparrow Terrace be fitted to one of the bungalows. Sparrow Terrace should ideally be positioned as high up as possible on the walls of the chosen building. The above enhancement would facilitate the aims and objectives of the UK BAP Species Action Plan for House Sparrows.

1 X 3 bay sparrow terrace     Schwegler 1SP Sparrow Terrace

Swifts spend just four months in Europe over the summer and during this time they use urban areas around our homes and buildings to provide nesting sites. Renovation and rebuilding of older houses means that many of these preferred spaces, such as under roofs, attics or tiles, are often blocked so that they cannot find sufficient nest sites. The Schwegler No. 17A or No. 17 Standard Swift Box is a good design (but any other similar brand or design would be suitable) and can accommodate three pairs of swifts in parallel and assists in the rapid formation of swift colonies. It is recommended that a single 3 bay swift box be be fitted on the wall of one of the bungalows. The box should be fitted as high as possible on the wall of the chosen building.

1 X Schwegler three bay swift brick

Note on breeding birds and vegetation clearance

It is strictly illegal in the UK to take, damage or destroy the nest of any wild bird while it is being built or in use. Therefore prior to any vegetation clearance or tree felling operation during the nesting bird season [March to August inclusive] trees and dense brush should be carefully inspected for any active nests. If an active nest is identified this area should be left undisturbed until nesting activity has finished. It is an offence to disturb any nesting bird or its nest. It is recommended that if vegetation clearance operations are scheduled to take place during the nesting bird season then a breeding bird survey should first be undertaken by a suitably qualified person in order to asses any potential impacts and inform future action. If active nests are found an exclusion area should be established around the nest in order to ensure that birds are not disturbed until nesting activity has ceased.

Enhancements for hedgehogs and other small mammals

The State of Britain’s Hedgehogs 2022 report reveals hedgehogs have declined by between 30% – 75% across different areas of the countryside since 2000.

Grace Johnson, Hedgehog Officer, Hedgehog Street (a joint campaign by BHPS and PTES) says:

“Hedgehogs can travel around one mile every night through gardens and parks in search of food and mates. It’s clear from our report that gardens can be havens for hedgehogs, but only if they are connected via gaps in or under garden boundaries to let hedgehogs in and out. A ‘Hedgehog Highway’ (a 13cm or CD case sized square gap) will enable hedgehogs to roam between neighbouring gardens and green spaces, which is vital to their survival.”

Hedgehog passes should be incorporated into fence lines of the development  (an  (approximately 10x10cm gap at the bases of fences). Although no evidence for hedgehogs was found during the survey, they could be present at the site. The site is certainly suitable for hedgehogs and given the current alarming declines in hedgehog numbers consideration should be  given to implementing the following guidelines designed to protect hedgehogs during development projects.

+ All fencing erected between residential properties should contain at least one gap at ground level (approximately 10x10cm) in order to allow movement of mammals, such as hedgehogs and amphibians, such as common frogs, to move freely between gardens.

+ Several boards or planks should be used to provide an escape routes out of any excavations at the end of each working day to facilitate a means of escape for mammals such as hedgehogs.

6.0 Defra Biodiversity Metric calculations for the proposed development at Aspinall Street in Heywood with required habitat improvements and planting instructions.

The Defra Biodiversity Metric is designed to provide a means of assessing changes in biodiversity value (losses or gains) brought about by development or changes in land management. The metric is a habitat based approach to determining a proxy biodiversity value. Baseline conditions at the Site were measured with areas (hectares - ha) calculated for each of the existing habitat parcels present. Proposed habitats to be ‘created’ were measured in the same way.

These measurements were input into the Biodiversity Metric 3.0 calculation tool along with the habitat type and a number of other variables including habitat condition and connectivity. Based on these multipliers the calculator provides a total number of biodiversity ‘habitat units’ for each habitat and an overall ‘net unit’ and ‘net percentage’ change for the whole site. Use of the tool therefore characterises the overall biodiversity interest of the existing and proposed habitats at the Site quantitatively and provides an indication of the overall loss or gain in biodiversity value at the Site.

Results from Biodiversity Metric 3

| Phase 1 habitat type | UK habs/broad habitat types | UK habs/habitat types | Area ha | Distinctiveness | Condition | Ecological connectivity | Strategic significance | Habitat units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tall rank grass | Modified grassland | Improved grassland | 0.075 | Medium 4 | Poor 1 | Low 1 | Within area formally identified in local strategy | 0.3 |
|  |  |  |  |  |  | Existing biodiversity value = 0.4 biodiversity habitat units Currently the development would cause a loss of 0.4 biodiversity habitat units | The proposed development must provide at least a 10% biodiversity gain. | 0.3 +10% =0.33 biodiversity habitat units |
| Hedgerow planting with trees | Native hedgerow with trees | hedgerow | 0.5 metres | Low | Moderate | Low | Not within areas identified in local strategy | This would provide 0.27 biodiversity habitat units |
| Urban vegetated garden | Urban vegetated garden | Urban vegetated garden | 0.03 hectares | Low | Moderate | Low | Not within areas identified in local strategy | This would provide 0.06 biodiversity habitat units |
|  |  |  |  |  |  |  |  | This gives a total of 0.33 biodiversity habitat units post-development which gives the required 10% uplift in biodiversity. |

In order to achieve the required minimum 10% uplift in biodiversity units post development it is suggested that a 50 metre length of native hedgerow with small trees of uk provenance be planted around the boundaries of the site. Planting a hedgerow with 'Poor' distinctiveness (ie low species diversity 2 to 3 tree or shrub tspecies) would provide the necessary 1.58 units; this would provide a  10% increase in biodiversity habitat units post development. A species rich hedgerow with 6 or more tree or shrub species would require only about 35 metres of hedging. In addition to hedges a further 200 square metres of the site will consist of planet gardens giving an additional 0.06 of a habitat unit. This gives a total of 0.33 habitat units which meets the required 10% increase in biodiversity post-development.

Hedgerow creation

The new hedgerow would be located along the boundary of the  development. The hedge can either be instead of a boundary fence or in addition to a boundary fence, whichever is deemed more appropriate. It is proposed that approximately 50 m of species-poor hedgerow be created (1 to 3 species) as part of the new habitat ; or  alternatively approximately 30 metres of species rich hedgerow (6 plus species).

The aim is to create habitats that are an improvement on the existing habitats. These will be suitable to support a wide range of species, including breeding birds, amphibians, foraging bats, invertebrate species and other small mammals.

Hedgerow specification

Areas of hedgerow to be restored should use typical native species of local provenance. The following list of species (below) are typical of hedgerows and woodland in Greater Manchester and planting should contain a mix of the following species, with a predominance (50% +) of hawthorn (Crataegus monogyna) and or blackthorn (Prunus spinosa).

Beech spp (up to 10%) Fagus species

Hornbeam (5% +) Carpinus betulinus

Climbing  roses (5% +) Rosa species

Rowan (5% +) Sorbus aucuparia

Holly (10% +)Ilex aquifolium

Hazel (10% +) Corylus avellana

Hawthorn/Blackthorn (50% +) Crataegus monogyna / Prunus spinosa

Cotoneaster and Pyracantha (10% +) Cotoneaster and Pyracantha species

Ground flora/climber may include:

any of the following

Bluebell (UK variant) Hyacinthoides non-scripta

Honeysuckle Lonicera periclymenum

Red campion Silene dioica

Ramsoms (wild garlic) Allium ursinum

Wood anemone Anemone nemorosa

Primrose Primula vulgaris  or other primulas

Herb-robert  Geranium robertianum

Wood avens Geum urbanum

Common dog-violet  Viola riviniana or pansys

All hedgerow planting shall be carried out in accordance with BS4428:1989 code of practice for general landscape operations (excluding hard surfaces). Hedgerow plants shall either be notch or trench planted into uncompacted soil, free of vegetation. Hedge trenches shall be dug to a minimum depth of 400mm and width of 600mm, with the plants put into the ground at the same depth at which they had been previously grown in the nursery. All plants shall be well heeled in after planting. Hedging plants to be supplied as bare-root specimens. All plant roots shall be kept under a moist cloth or in a bucket of water during planting. Plants not planted on day receipt are to be heeled-in in a pre-prepared trench and kept moist until plants are required. Hedges should only be planted between November and March, while plants are still dormant. Avoid planting in very cold or wet weather. All plants to be feathered whips 1.25  1.50 m high, to be planted at 0.3 m spacing in a double row at 0.4 m apart.

The following species can be planted in the hedge bases at a rate of 2 to 4 plants per metre of hedge to further improve diversity.

Common name

Herb Robert or other geranium species (Geranium robertianum)

Red campion (Silene dioica)

Ivy (Hedera helix)

Periwinkle (Vinca minor or major)

Dog violet  (Viola riviniana) or pansys

White campion (Silene alba)

Honeysuckle (Lonicera sp)

Tutsan species (Hypericum sp)

Primulas (Primula sp)

Add trees and shrubs to gardens

Add small garden trees and shrubs to the planting scheme. Where possible each garden should include one of the listed berry bearing trees which will provide useful autumn forage for birds. Spring flowering shrubs from the list below should be added to the planting scheme scheduled for the site at a rate of 2 plants per garden. Flowering currants provide a much needed source of nectar to bees in early spring.

Beneficial trees and shrubs for gardens in Greater Manchester

| Common name | Latin name |
| --- | --- |
| Shrubs and trees suitable for gardens |  |
| Rowans & whitebeams | Sorbus sp |
| Cherries and plums | Prunus sp |
| Apples and crabapples | Malus sp |
| The species below may be used either as garden plants or in hedges |  |
| Guelder rose or other viburnums | Viburnum sp |
| Flowering currant or other currant species | Ribes sp |
| Hawthorn | Crataegus monogyna |
| Blackthorn | Prunus spinosa |
| Holly | Ilex aquifolium |
| Dogwoods | Cornus sp |

Similar species which will perform the same function may be substituted if necessary.

7.0 CONCLUSION

In order to achieve the required minimum 10% uplift in biodiversity post development it is suggested that a 50 metre length of native hedgerow of uk provenance be planted around the boundaries of the site. The hedge can either be instead of a boundary fence or in addition to a boundary fence, whichever is deemed more appropriate. In addition to hedges a further 200 square metres of the site will consist of three gardens  each with at least two small native trees or shrubs planted in it. This will give the required 10% increase in biodiversity at the site post-development. The addition of two bat roosting boxes and a sparrow nesting terrace, and a swift nesting terrace will further improve biodiversity. Taken together the measures will benefit all the protected species at the site and improve the overall biodiversity on the site.

Prepared by

Richard Parkinson BSc[Hons] FLS MCIEEM

Principal Ecologist – Living Ecosystems

www.livingecosystems.co.uk

June 2023

8.0 References

BTCV (2001) Woodlands a practical handbook, Printed by Watkins Studios Ltd, Biggleswade, Beds.

ENGLISH NATURE (1998). UK Biodiversity Group Tranche 2 Action Plans. Volume 1 – Vertebrates and Vascular Plants. English Nature, Peterborough.

ENGLISH NATURE (2004) Bat mitigation Guidelines - working today for nature tomorrow, English Nature Peterborough.

ENGLISH NATURE (1993) Linear Features: linear habitats and wildlife corridors No. 60 English Nature Research Reports, English Nature Peterborough.

EOAC. (1979). Categories of Breeding Bird Evidence. European Ornithological Atlas Committee.

GOVERNMENT CIRCULAR: BIODIVERSITY AND GEOLOGICAL CONSERVATION – STATUTORY OBLIGATIONS AND THEIR IMPACT WITHIN THE PLANNING SYSTEM (PPS9). (2005). DEFRA.

JNCC (Revised reprint, 2003) Handbook for Phase 1 habitat survey – a technique for environmental audit, JNCC.

KIRBY.P. (1992). Habitat Management for Invertebrates: a practical guide. JNCC.

RODWELL, J.S. (1991) British Plant Communities Volume 1 – Woodlands and scrub, Cambridge University Press.

RODWELL, J.S. (1991) British Plant Communities Volume 2 – Mires and heaths, Cambridge University Press.

RODWELL, J.S. (1992) British Plant Communities Volume 3 – Grasslands and montane communities, Cambridge University Press.

RODWELL, J.S. (1995) British Plant Communities Volume 4 – Aquatic communities, swamps and tall-herb fens, Cambridge University Press.

RODWELL, J.S. (2000) British Plant Communities Volume 5 – Maritime communities and vegetation of open habitats, Cambridge University Press.

STACE, C.  (1997)  New Flora of the British Isles.  Second edition, Cambridge University Press.

STEWART, A., PEARMAN, D.A. & PRESTON, C.D. (EDS) (1994) Scarce Plants in Britain.  JNCC

SUTHERLAND W.J & HILL D.V (1995) Managing Habitats for Conservation. Cambridge University Press.

THE REGIONAL BIODIVERSITY STEERING GROUP FOR NORTH WEST ENGLAND. (1999)  A Biodiversity of North West England Volumes 1 & 2.

WATSON.D, HACK.V, FASHAM.M. (2000). Wildlife management & habitat creation on landfill sites. Ecoscope.

WIGGINTON, M.J. (ED) (1999) British Red Data Books: 1 Vascular Plants.  (Third edition).