# Highways and Engineering Service - Economy and Environment

# Drainage Consultation on Application – 24/01086/FUL - 
Lower Town House Barn Townhouse Road Littleborough OL15 9JX

LLFA Consultation Response on behalf of Head of Highways and Engineering

Date of Response: 19th May 2025

Location & Proposed Development

Lower Town House Barn Townhouse Road Littleborough OL15 9JX

Change of use from existing barn to 2no' residential dwellings and ancillary external alterations and landscaping

Documents Reviewed

- Flood Risk Assessment, doc ref. 070722.docx

- Existing Site Plan, drawing no. 02

- Proposed Site Plan, drawing no. 04

- EA Consultancy Response

Drainage & Flood Officer Comments

- No drainage proposals have been provided. The application should be supported with a drainage strategy, design and layout for both foul and surface water. Together with a completed SuDS Proforma. The proposed site is located within Littleborough, which is designated as a Critical Drainage Area.

- The applicant has provided a Flood Risk Assessment which identifies that the site is within Flood Zone 2. The FRA identifies the 100-yr +35% flood level as 157.17m. The existing ground levels are indicated as approx 158.00m. However, the site plan shows ground levels of approx 100.00m. The FFL for the properties should be 300mm above the max flood level. In addition, the EA have challenged the flood levels in the report and these points should be clarified.

- The FRA states there is no flood risk from reservoirs. However, it is not clear if the local reservoir is incorporated in the EA flood routing plan. The condition of the dam should also be investigated. A more detailed reservoir flood risk assessment and site flood routing plan should be provided.

- The above points should be addressed before the application is approved.

Consultation by Keith Mealey

Drainage & Flooding Officer

- Mob 0787 2500 722 / Office 01706 924657

keith.mealey@rochdale.gov.uk