Knacks Farm riding arena planning application 23/00363/FUL

Heritage statement

Summary of changes to the lighting proposal:

Original	Revised

Height of poles	8m	6m

Number of poles	6	10

Lamps (each)	610w	200w

Lux (overall average)	200	130

Uniformity	0.75	0.60

Statement:

Whilst Knacks Farm itself is in a Non-Designated Heritage Assets zone, there are two Designated Heritage Assets, namely Catley Lane Head Conservation Area and Prickshaw/Broadley Fold Conservation Area, which could potentially be impacted by the lighting included in the application. Taking this into account the lighting plan has been re-considered and modified in an effort to reduce any possible impact on the surrounding area by:

- Lowering the lighting poles (8m to 6m)

- Reducing the power of each lamp (610w to 200w)

- Lowering the overall level of lighting (average 200lux to 130lux)

- Lowering the uniformity of light (0.75 to 0.60)

Whilst the above changes do reduce the overall effectiveness and uniformity, even after adding the four additional poles this is considered to be a reasonable compromise given the facility is for personal use only. Any further lowering of the overall effectiveness would take the resultant lighting below the minimum level set by the British Horse Society and therefore deem the facility unfit for purpose.

Retractable lighting poles, as suggested in the “Conservation Consultation Response” have been investigated however this type of equipment is typically used in income generating sports stadiums where the very significant additional cost can be incorporated into the business model. As this arena is limited to personal use only, the cost of the equipment is deemed “out of proportion” and prohibitive.