| Client: | #### Rochdale MBC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: | ### MCF Castleton |  |  |  |  |
| Subject: | # Manchester Road / Nixon St - Traffic Impact Assessment |  |  |  |  |
| Doc: | MCFNixonSt_TIA_CAT_050321 |  |  |  |  |
| Author: | ##### Charles Taylor |  |  |  |  |
| Date: | 5th March 2021 |  |  |  |  |
| Ref: | X5/043 |  |  |  |  |
| Contact: |  |  | E-Mail: | Tel: |  |
| Ed Coventry |  |  | ed.coventry@TfGM.com | 0161 244 1876 |  |
| Charles Taylor |  |  | charles.taylor@TfGM.com | 0161 244 1800 |  |

- Introduction

  - The following report considers the traffic impact resulting from the proposed changes to the layout and operation of the junction of Manchester Road / Nixon St, as illustrated in Appendix A.

  - The proposals involve modification of the Manchester Road / Nixon St junction, to incorporated controlled segregated pedestrian and cycling facilities on all arms.

  - To achieve the above arrangement, the left turn flare lane on Nixon St northbound has been omitted.

  - The following considers the likely operational efficiency of the junction during the morning peak and evening peak periods, in the assessment year.

- Junction Layout and Signal Stage Sequences

  - The proposed junction layout is included in Appendix A.

  - Manchester Rd / Nixon St – Existing Staging

  - Manchester Rd / Nixon St – Proposed Staging

- Phases E, F, G are cycle phases; phases H, I, J are pedestrian phases.

- Traffic Flows

  - The counts have been derived from surveys commissioned by Highways Forecasting and Analytical Services (HFAS) on 28th November 2019.

- Modelling Details

  - The junction has been modelled using LINSIG (version 3.2.4) for the weekday morning and evening peak periods, in the assessment year.

  - The existing maximum cycle time of 72 seconds has been used throughout.

  - The following scenarios have been modelled:

    - Existing junction layout and staging.

    - Proposed junction layout and staging.

- Results of Modelling

  - The table below indicates the predicted degree of saturation, queue length and delay for each modelled scenario.

  - The degree of saturation is a measure of how close to capacity a link is predicted to operate, with 90% taken to be the maximum practical level (between 90% and 100% delay begins to increase exponentially).

  - Queues are mean maximums and will be exceed for half the modelled period (where a link is at practical capacity the queue could be up to twice indicated).

  - Morning Peak Period

| Link Description | Existing Cycle Time 72s |  |  | Proposed Cycle Time 72s |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Degree of saturation (%) | Delay (s/pcu) | Queue length (metres) | Degree of saturation (%) | Delay (s/pcu) | Queue length (metres) |  |
| Nixon Street Right and Left | 62 | 45 | 24 | 75 | 59 | 28 |  |
| Manchester Road Southbound Ahead and Right | 42 | 8 | 29 | 60 | 20 | 49 |  |
| Manchester Road Northbound Ahead and Left | 61 | 15 | 58 | 80 | 26 | 80 |  |
| Total Delay (pcu-hours) | 7 |  |  | 11 |  |  |

  - Evening Peak Period

| Link Description | Existing Cycle Time 72s |  |  | Proposed Cycle Time 72s |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Degree of saturation (%) | Delay (s/pcu) | Queue length (metres) | Degree of saturation (%) | Delay (s/pcu) | Queue length (metres) |  |
| Nixon Street Right and Left | 57 | 53 | 16 | 57 | 53 | 16 |  |
| Manchester Road Southbound Ahead and Right | 32 | 6 | 18 | 47 | 17 | 34 |  |
| Manchester Road Northbound Ahead and Left | 62 | 12.4 | 59 | 83 | 27 | 91 |  |
| Total Delay (pcu-hours) | 5 |  |  | 10 |  |  |

- Conclusion

  - The results of the modelling indicate that the proposed changes to the layout and operation of the junction of Manchester Road with Nixon Street would result in a small reduction to junction efficiency in both peak periods.

  - This should, however, be balanced against the significant qualitative benefits afforded to pedestrians and cyclists.

  - The modelling assumes that the pedestrian stage appears in every signal cycle - this represents a worst-case scenario. In reality the stage may be called less frequently.

  - There are additional delay benefits afforded by adaptive signal control such as SCOOT or MOVA that have not been considered by this fixed-time modelling.

  - The above assessments are based on surveys undertaken prior to the COVID crisis. It is not yet clear what the long-term effects of COVID will be on traffic demand and distribution in this location.

- Appendix A – Manchester Rd / Nixon St – Proposed Layout