|  |  | Public Health and Communities Kuiama Thompson Director of Public Health and Communities # Public Protection and Customers Number One Riverside, Smith Street, Rochdale, OL16 1XU Tel: (01706) 924688 Email: jonathon.parr@rochdale.gov.uk Website: www.rochdale.gov.uk |
| --- | --- | --- |

Date: 28/01/2026

Team: Environmental Control (Landfill Gas)

Application Number: 25/00484/FUL

Proposal: Construction of a row of 4 no. 3 storey, 3 bed terraced dwellings, Land Between 22 And 34 Durham Street Rochdale, OL11 1JJ X.

Dear Nick,

I have reviewed the site history, and evaluated this against the proposal.

The site has been open space since the earliest OS maps it is not until the 1950s that development is noted, which appears to be in the form of garages.  From the 2005 Google Earth StreetView image, these appear to have asbestos cement roofs.  They appear to have been demolished sometime between 2009 and 2012.  A review of aerial photos would indicate that the asbestos may have been left in situ, and fly tipping may have occurred at the property.

Because of this, and because of the potential sensitive receptors which may be present on site, then I would request the following condition, should you grant permission:

“Prior to commencement of the development a Phase I Report (Preliminary Risk Assessment) must be completed to assess the actual and/or potential contamination risks at the site. The Phase 1 Report must be submitted to, and approved in writing by, the Local Planning Authority and shall include a desk top study, site walk over, conceptual model, basic hazard assessment and recommendation regarding the need or otherwise for further investigation and risk assessment.

Where the Phase 1 Report has identified potential unacceptable risks, a suitable investigation, risk assessment and remedial action shall be carried out in accordance with details and within a timescale to have first been submitted to and approved in writing by the Local Planning Authority.

In the event that contamination is suspected or found at any time when carrying out the approved development that was not previously identified, or contamination is caused, the Local Planning Authority shall be notified immediately and development affected and potentially affected by the contamination must stop. An investigation and/or risk assessment and/or remediation shall be carried out in accordance with an agreed process and within agreed timescales to have first been submitted to and approved in writing by the Local Planning Authority. If contamination is not suspected, found or caused during development, this must be confirmed in a completion letter which is subject to the approval in writing of the Local Planning Authority.”

If you have any queries, please do not hesitate to reach out.

Yours sincerely,

Jonathon Parr

Jonathon Parr

Environmental Health Senior Officer (Environment Prot)