# Highways and Engineering Service - Economy and Environment

# Drainage Consultation on Application – OTH/2025/1313 – Ramsden Farm, Roundthorn Road, Middleton, Manchester, M24 1FT

LLFA Consultation Response on behalf of Head of Highways and Engineering

Date of Response 25th February 2026

Location & Proposed Development

Ramsden Farm, Roundthorn Road, Middleton, Manchester, M24 1FT

Discharge of conditions 9 (Drainage), of planning permission 24/00924/FUL.

Documents Reviewed

OTH/2025/1313

- Optimized Layout Plan

- Drainage Strategy

24/00924/FUL

- Location Plan

- Drainage & Site Design Plan

- Proposed Site Plans

- Soakaway Investigation ref: 744045-SI-2 dated 13/01/2025

- United Utilities SafeDig

- Previous LLFA comments: Consultee Response dated 13/11/2024

Drainage & Flood Officer Comments

- The application contains an updated drainage strategy. It addresses the points raised in response to 24/00924/FUL. It includes a brief FRA, supporting report, hydraulic calculations and infiltration report as was required from the initial Full application.

- The aim should be to discharge surface run off as high up the following hierarchy of drainage options as reasonably practicable: The ordered drainage techniques to be assessed with the aim to reduce or omit lower techniques:

- Control at Source (Infiltration / Recycling)

- On Site Treatment (Basins, Swales, Rain gardens, Tree pits)

- Local Treatment (Adoptable Ponds, Basins, Swales etc)

- Regional Treatment (ditto usually to a larger scale)

- Discharge to Watercourses / Bodies

- Discharge to SW Sewers

- Discharge to Combined Sewers

- The drainage strategy references a Soakaway Investigation ref: 744045-SI-2 which has since been uploaded as an attachment to 24/00924/FUL. Site-specific BRE365 infiltration tests have been undertaken and finds 3.00x10-5 m/sec confirming suitability for soakaway design.

- A maintenance plan and schedule is still required to ensure the long-term performance and effectiveness of the planned soakaway features.

- Calculations are provided showing no instances of flooding of the surface water drainage system for a 1-in-100-year storm event + 45% climate change allowance, appropriate for the ‘Irwell Management’ Catchment area.

- The change in the drainage design from the initial Full application is minimal with the removal of the soakaway in plot 6 being the only alteration. This is as a result of the acknowledgement of infiltration testing identifying unsuitable made ground in this plot.

Drainage & Flood Officer Summary:

- The principles of the drainage design remain consistent with the original Full application and all points that were raised have been addressed.

- A maintenance & management plan should be submitted for the drainage system in general and the soakaways in particular. However, this does not need to be a pre-commencement requirement.

- It is considered that condition 9 (Drainage) can be discharged.

Consultation by Keith Mealey

Drainage & Flooding Officer

keith.mealey@rochdale.gov.uk