# Highways and Engineering Service - Economy and Environment

# Drainage Consultation on Application –

# PL/2025/1215- LEOPOLD COURT

LLFA Consultation Response on behalf of Head of Highways and Engineering

Date of Response: 2nd March 2026

Location & Proposed Development

LEOPOLD COURT, LEOPOLD STREET, ROCHDALE, OL11 5HA

Minor extensions and infills to facilitate the re-organisation and refurbishment of internal spaces and to increase bedroom capacity from 23 to 32 (29 standard, 2 category 3 and one emergency room), along with replacement of windows and doors, resurfacing of external areas, minor landscape works, new access gates and new and upgraded external lighting.

Documents Reviewed

- Asset Condition Survey Report

- CCTV Drainage Survey

- Existing Site Layout and Block Plan

- Proposed Building Site and Site Sections

- Proposed Building Elevations

- Site Location Plan

- Topographical Survey

- UU Safedig Records

- EA Flood Maps for Planning

Drainage & Flood Officer Comments

- Limited drainage information has been provided. The development is less than one hectare and is entirely located within flood zone 1. When reviewing flood maps for planning, no area of the site shows surface water flooding.

- The aim should be to discharge surface run off as high up the following hierarchy of drainage options as reasonably practicable: The ordered drainage techniques to be assessed with the aim to reduce or omit lower techniques:

- Control at Source (Infiltration / Recycling)

- On Site Treatment (Basins, Swales, Rain gardens, Tree pits)

- Local Treatment (Adoptable Ponds, Basins, Swales etc)

- Regional Treatment (ditto usually to a larger scale)

- Discharge to Watercourses / Bodies

- Discharge to SW Sewers

- Discharge to Combined Sewers

- The proposal’s design feasibility should be based on a 1-in-100-year storm event with a 45% allowance for climate change, which is appropriate for peak rainfall. This climate allowance is appropriate for the ‘Irwell Management’ Catchment area for ‘More Vulnerable’ vulnerability class.

- However, given the minor nature of the proposals it is assumed that any drainage will be discharged to the existing infrastructure. It is unclear if the main building incorporates any sustainable drainage, but the scope of the works is not considered to warrant investigating this further.

Drainage & Flood Officer Summary:

- No drainage proposals have been submitted. However, given the minor nature of the proposals it is assumed that any drainage will be discharged to the existing infrastructure and we have no further comment.

Consultation by Keith Mealey

Drainage & Flooding Officer

keith.mealey@rochdale.gov.uk