# DESIGN AND ACCESS STATEMENT

# 34 Rooley Moor Road

## Design Principles

Although a Design and Access Statement is not always mandatory for Listed Building Consent, this section is provided in support of the application.

### Amount

The proposal involved replacement of existing windows and doors only. No extensions or enlargements were undertaken.

### Layout

There were no changes to internal layout or building footprint. All existing openings were retained without alteration.

### Scale

The scale of all openings and joinery elements remains unchanged. Replacement windows replicate the dimensions of those removed.

### Appearance

The design approach was conservation-led and based on faithful replication.

- Window proportions and glazing patterns were retained.

- Faux lead detailing was reinstated on the principal elevation.

- Timber frames were painted to match the previous appearance.

- Front and rear doors are now traditional hardwood stable doors, replacing modern unsympathetic elements.

The visual character of the building within the terrace remains unchanged or enhanced.

### Materials

- Windows: Sapele hardwood, painted finish, 14mm slimline double glazing.

- Doors: Painted hardwood stable doors.

Traditional painted timber was deliberately chosen to maintain material authenticity and visual continuity.

## Access

### Site Access

Access arrangements to the property remain unchanged. The works did not alter entrance positions or thresholds.

### Inclusive Access

There has been no change to accessibility arrangements. The proposal does not affect level access, thresholds, or circulation.

As this is a like-for-like replacement of joinery only, accessibility considerations remain as existing.

# Conclusion

The window and front and rear door replacements undertaken in 2024 were necessary due to significant deterioration, water ingress and safety concerns.

The works:

- Replaced non-original twentieth-century elements,

- Faithfully replicated previous window detailing including faux lead,

- Retained all historic masonry openings,

- Used appropriate painted hardwood materials,

- Enhanced the façade by removing unsympathetic modern doors,

- Introduced no structural alterations.

The proposal preserves the special architectural and historic interest of the Grade II listed building in accordance with statutory duties and national policy and results in no harm to its significance.

Retrospective Listed Building Consent is therefore respectfully sought.