# Highways and Engineering Service - Economy and Environment

# Drainage Consultation on Application – PL/2026/0352 – Land adjacent to 8 Top Schwabe Street, Middleton, M24 4TQ

LLFA Consultation Response on behalf of Head of Highways and Engineering

Date of Response: 12th July 2026

Location & Proposed Development

Land Adjacent to 8 Top Schwabe Street, Rhodes, Middleton, M24 4TQ

Outline application (including access) for 1no’ detached 4-bedroom dwelling.

Documents Reviewed

- Location Plan

- Existing Site Layout

- Design and Access Statement

- Proposed Site Layout

- United Utilities Safe Dig Maps

- EA Flood Maps for Planning

Drainage & Flood Officer Comments

- No detailed drainage information has been provided. The site layout indicates a single pipe connection to an existing main but it is unclear if this is foul, surface water or combined. Outline planning proposals should include an outline drainage strategy and plans.

- The aim should be to discharge surface run off as high up the following hierarchy of drainage options as reasonably practicable: The ordered drainage techniques to be assessed with the aim to reduce or omit lower techniques:

- Control at Source (Infiltration / Recycling)

- On Site Treatment (Basins, Swales, Rain gardens, Tree pits)

- Local Treatment (Adoptable Ponds, Basins, Swales etc)

- Regional Treatment (ditto usually to a larger scale)

- Discharge to Watercourses / Bodies

- Discharge to SW Sewers

- Discharge to Combined Sewers

- Infiltration potential should always be considered, with support from site-specific ground investigations and infiltration testing in accordance with BRE365. Garden soakaways and permeable paving could be accommodated. For a plot of this scale, SuDS source controls can also be incorporated including hydroplanters at rwp downpipes. The garden area also allows the possibility for landscaped ponds and basins.

- UU records indicate there is a watercourse along the northern site boundary and a public surface water sewer in the highway.

- The applicant’s design feasibility should be based on a 1-in-100-year storm event with a 45% allowance for climate change. This climate allowance is appropriate for the ‘Irwell Management’ Catchment area for ‘Highly’ vulnerability class.

Drainage & Flood Officer Summary:

- Insufficient drainage information has been provided.

- An outline application should incorporate an outline drainage strategy which sufficiently considers each drainage technique of the hierarchy.

- The above points should be addressed before the application can be approved.

Consultation by Keith Mealey

Drainage & Flooding Office

keith.mealey@rochdale.gov.uk