# Highways and Engineering Service - Economy and Environment

# Drainage Consultation on Application – OTH/2026/0488 – Land West of New Street, Littleborough, OL15 8HW

LLFA Consultation Response on behalf of Head of Highways and Engineering

Date of Response: 16th July 2026

Location & Proposed Development

Land west of New Street, Littleborough, OL15 8HW

Submission of details to comply with conditions 9 (WSI), 10 (surface water drainage), 11 (method statement control of invasive plants), 12 (CMS) and 13 (CTMP) on planning permission 22/01039/FUL.

Documents Reviewed

OTH/2026/0488

- Covering Letter

- FRA and Outline Drainage Strategy Rev. 02

- Proposed Drainage Plan Sheets

- S104 Drawings

22/01039/FUL

- Decision Notice

- Drainage Strategy Plan

- FRA and Outline Drainage Strategy

- Phase I Geoenvironmental Site Assessment ref. 15-075-R1-2

- Proposed Site Layout

Drainage & Flood Officer Comments

- The proposed drainage plan remains consistent with the original 22/01039/FUL application. But the latest submitted ‘Flood Risk Assessment and Outline Drainage Strategy’ remains an unchanged revision from the original application. Foul and surface water is treated separately through a piped network to serve the estate, with a piped gravity surface water system feeding to an attenuation basin before discharging into the River Roch.

- The aim should be to discharge surface run off as high up the following hierarchy of drainage options as reasonably practicable: The ordered drainage techniques to be assessed with the aim to reduce or omit lower techniques:

- Control at Source (Infiltration / Recycling)

- On Site Treatment (Basins, Swales, Rain gardens, Tree pits)

- Local Treatment (Adoptable Ponds, Basins, Swales etc)

- Regional Treatment (ditto usually to a larger scale)

- Discharge to Watercourses / Bodies

- Discharge to SW Sewers

- Discharge to Combined Sewers

- No assessment of infiltration using site based investigation has been undertaken. Therefore infiltration cannot be excluded as a potential option at this stage

- There is no SuDS source control for the highways, access road or private areas. The reasons provided in the FRA&DS are largely based on spatial constraints which is not acceptable. Space for water should be a design consideration.

- Highways being offered for adoption will be required to incorporate Highway SuDS as part of any s38 application.

- UU may also require two stage treatment of highways runoff and incorporation of SuDS in systems being offered for adoption. Furthermore, they may refuse to accept a piped system which provided for the 100 year event if no SuDS are provided.

- The initial section of drainage in the access road is only receiving highway runoff and therefore it may need to be offered for highways adoption.

- There is no indication if any agreements in principle, technical approvals etc have been obtained from or discussed with adoption bodies (eg UU, RBC Highways).

- Microdrainage calculations have been performed to a 40% climate change factor. Design feasibility should be based on a 1-in-100-year storm event with a 45% allowance for climate change. This allowance is appropriate for the ‘Irwell Management’ Catchment area.

Drainage & Flood Officer Summary:

- The proposals do not incorporate a suitable investigation of infiltration potential.

- Green SuDS source control is required across the development area and along highways

- A 45% climate change allowance should be used in the Microdrainage model.

- The adoption strategies and feedback from adopting bodies should be provided.

- The above points should be addressed before the condition can be discharged.

Consultation by Keith Mealey

Drainage & Flooding Office

keith.mealey@rochdale.gov.uk